# WORK WRITE-UP AND COST ESTIMATE (SAMPLE)

Applicant:

Mrs. Beaslev

425 Fourth Street

Application:	#129

Inspector:

Orry Marrinoux	Date Inspected:

Estimated Cost

Living Room	10' x 15'

Floors	Remove existing sub floor (approximately 210 square feet) and	$	 replace with plywood sub floor at least 5/8".

Install sheet vinyl over wood surface, "Armstrong Standard" or	$	 equal (approximately 210 square feet).

Walls	Patch holes and cracks in existing plaster (approximately 6	$	 square feet). Include lathing on south and west walls.

Paint walls (approximately 480 square feet) with two coats of interior	$	 latex paint, "Sherman Williams Good" or equal with roller.

Bedroom #2	9' x 11'

$

Window	Install screen and lock (3' x 4”)

Closet Walls

Doors	and Woodwork

Hall

Install 16" x 36" #2 pine shelf and 1-3/8" clothes pole.	$

Paint with 2 coats of interior latex "Sherman Williams Good" or	$	 equal with roller.

Paint using semi gloss "Sherman Williams Good" or equal using brush.	$

4' x 10'

Floor	Remove existing sub floor (approximately 40 square feet) and replace	$	 with plywood sub floor at least 5/8".

Install vinyl asbestos floor tile "Armstrong Standard" or equal	$	 (approximately 40 square feet).

Hall (cont'd.)

Ceiling

Install shoe and baseboard (approximately 20 linear feet). Install frame and cover on access hole (approximately 3’ x 3’).

Estimated Cost

$

$

Electric	Remove existing hanging light cord and replace with NL approved	$	 ceiling fixture and switch.

Closet

Walls

Kitchen

Rebuild existing closet 3'9” x 2'0"; door size 2068; install 12" #2	$	 pine shelf and 1-3/8" clothes pole.

Apply 2 coats interior latex “Sherman Williams Good” or equal with	$	 roller (approximately 200 square feet).

Apply semi-gloss paint  to  doors,  door  jambs  and   base  board.	$	 “Sherman Williams Good” or equal with brush.

11'6” x 14’5”

Install shoe and baseboard approximately 50 linear feet.	$

Apply 2 coats interior latex, "Sherman Williams Good" or equal with	$	 roller approximately 250 square feet.

Ceiling	Install 12" x 12" Celotex tile over existing wood ceiling	$	 (approximately 165 square feet) with adhesive.

Windows	Remove existing wood windows, reset and install new trim	$	 approximately 2' x 3'.

Paint window trim using semi-gloss “Sherman Williams Good" or	$	 equal with brush.

Cabinets

Remove 3 base kitchen cabinets and countertop.	$

Install 36” door base cabinets with 2 drawers and 2 cabinets and	$	 Formica countertop and 4” backsplash (cost allowance $	).

Install 36” wall cabinets 24" high over base cabinet (cost	$	 allowance $	).

Kitchen (cont'd.)

Install stainless steel sink in top 32” x 21” (cost allowance $	).

Estimated Cost

$

Electric		Install 10" ventilating fan, vent fan through wall over stove.	$

Remove existing electrical panel box and install a new panel box (100	$	 amp. minimum with breaker switch) and place all wiring inside wall.

Replace all hazardous wiring as per code.

Install 2 new electrical outlets in kitchen, check wiring, replace as	$	 necessary.

Bathroom	6' x 8'0"

Floor	Remove existing sub floor (approximately 54 square feet) replace with	$	 plywood sub floor at least 5/8".

Install vinyl asbestos floor tile, "Armstrong Standard" or equal	$	 (approximately 48 square feet).

Ceiling

Walls

Toilet Exterior

Remove water damaged plaster down to lathes, replace lathes if	$	 damaged, and replaster (approximately 48 square feet), and paint with

2 coats of interior latex "Sherman Williams Good" or equal.

Scrape, seal and repaint all walls with 2 coats of interior latex	$	 “Sherman Williams Good" or equal.

Remove and replace existing toilet with new toilet. "Standard	$	 quality."

Joists	Remove 4 rear joists, size 2" x 10" x 14" and replace with same size	$	 joists.

Facia and Trim

Screen Doors

Remove 2 deteriorated barge rafters size 2” x 6” x 12’. Replace with	$	 same size rafters.

Replace 2 existing deteriorated screen doors 3' x 6’8” with new	$	 aluminum screen doors.

| Exterior (cont’d.) |  | Estimated Cost |
| --- | --- | --- |
| Exterior Windows Roof | Weatherstrip 8 windows with spring bronze. Remove existing asphalt roofing down to existing sheathing. Remove and replace sheathing if deteriorated. Install new asphalt/shingles/300 lb. Flash all valleys. Install new aluminum gutters and down spouts. | $ $ $ |